EXP 1a  To develop electromagnetic simulation models for TEM, TE and TM propagation modes in suitable microwave transmission structures.

Code:

%  EM SIMULATION MODELS FOR TEM, TE AND TM MODES

%  Structures : Air-filled coaxial line (TEM)

%               WR-90 rectangular waveguide (TE10 and TM11)

%  Run in MATLAB R2016b or later (no toolboxes needed).

%  ========================================================================

clc; clear; close all;

%% 1. INPUT PARAMETERS (from the lab sheet)

c    = 299792458;            % speed of light (m/s)

mu0  = 4e-7*pi;

eps0 = 1/(mu0*c^2);

eta0 = sqrt(mu0/eps0);       % free-space impedance (ohm)

f  = 10e9;                   % operating frequency (Hz)

a  = 22.86e-3;               % waveguide width  (m)

b  = 10.16e-3;               % waveguide height (m)

L  = 100e-3;                 % length (m)

ri = 1.5e-3;                 % coax inner radius (m)  -> about 50 ohm

ro = 3.45e-3;                % coax outer radius (m)

w  = 2*pi*f;

k0 = w/c;

E0 = 1;                      % field amplitude (normalised)

%% 2. THEORETICAL CUTOFF AND PROPAGATION CONSTANT

fc_fun = @(m,n) (c/2)*sqrt((m/a)^2 + (n/b)^2);

fc_TE10_th = fc_fun(1,0);

fc_TM11_th = fc_fun(1,1);

kc_TE10 = 2*pi*fc_TE10_th/c;

kc_TM11 = 2*pi*fc_TM11_th/c;

beta_TE10_th = getBeta(k0, kc_TE10);   % real if propagating

beta_TM11_th = getBeta(k0, kc_TM11);   % -j*alpha if evanescent

beta_TEM_th  = k0;                     % TEM: beta = k0, fc = 0

Z0_coax      = (eta0/(2*pi))*log(ro/ri);

%% 3. NUMERICAL "SIMULATION" OF CUTOFF (2-D finite-difference eigen solver)

%  TE modes : Neumann  boundary (dHz/dn = 0) -> TE10 is the lowest nonzero

%  TM modes : Dirichlet boundary (Ez = 0)    -> TM11 is the lowest

Nx = 64;  Ny = 29;

kc2_TE = fdEig(a,b,Nx,Ny,'neumann');

kc2_TM = fdEig(a,b,Nx,Ny,'dirichlet');

kc_TE10_sim = sqrt(kc2_TE(find(kc2_TE > 1, 1)));   % skip the zero eigenvalue

kc_TM11_sim = sqrt(kc2_TM(1));

fc_TE10_sim = c*kc_TE10_sim/(2*pi);

fc_TM11_sim = c*kc_TM11_sim/(2*pi);

beta_TE10_sim = getBeta(k0, kc_TE10_sim);

beta_TM11_sim = getBeta(k0, kc_TM11_sim);

dev = @(sim,th) abs(sim-th)/abs(th)*100;

%% 4. RESULTS TABLE

fprintf('\n=========== RESULTS AT f = %.1f GHz ===========\n', f/1e9);

fprintf('Coax Z0 = %.2f ohm, TEM: fc = 0, beta = k0 = %.2f rad/m\n\n', Z0_coax, beta_TEM_th);

fprintf('%-28s %12s %12s %10s\n','Parameter','Theory','Simulated','%% Dev');

fprintf('%-28s %12.4f %12.4f %10.3f\n','fc TE10 (GHz)', fc_TE10_th/1e9, fc_TE10_sim/1e9, dev(fc_TE10_sim,fc_TE10_th));

fprintf('%-28s %12.4f %12.4f %10.3f\n','fc TM11 (GHz)', fc_TM11_th/1e9, fc_TM11_sim/1e9, dev(fc_TM11_sim,fc_TM11_th));

fprintf('%-28s %12.3f %12.3f %10.3f\n','beta TE10 (rad/m)', real(beta_TE10_th), real(beta_TE10_sim), dev(real(beta_TE10_sim),real(beta_TE10_th)));

fprintf('%-28s %12.3f %12.3f %10.3f\n','alpha TM11 (Np/m)', abs(imag(beta_TM11_th)), abs(imag(beta_TM11_sim)), dev(abs(imag(beta_TM11_sim)),abs(imag(beta_TM11_th))));

fprintf('\nTE10 : f > fc -> %s\n', ternary(f>fc_TE10_th,'PROPAGATES (beta real)','EVANESCENT'));

fprintf('TM11 : f < fc -> %s\n', ternary(f>fc_TM11_th,'PROPAGATES','EVANESCENT (beta imaginary)'));

fprintf('Deviation limit <= 5%% : %s\n', ternary(max([dev(fc_TE10_sim,fc_TE10_th) dev(fc_TM11_sim,fc_TM11_th)])<=5,'MET','NOT MET'));

%% 5. TE10 FIELDS  (E0 sin(pi x/a), Hx, Hz)

x = linspace(0,a,70);  y = linspace(0,b,35);  z = linspace(0,L,120);

beta = beta_TE10_th;

ZTE  = k0*eta0/beta;

% 5a. cross-section (x-y) at z = 0

[X,Y] = meshgrid(x,y);

Ey_xy = E0*sin(pi*X/a);                 % no variation along y

Hx_xy = -Ey_xy/ZTE;

Hz_xy = (1j*pi*E0/(k0*eta0*a))*cos(pi*X/a);

% 5b. longitudinal (x-z) plane at y = b/2

[Z,Xz] = meshgrid(z,x);

phase  = exp(-1j*beta*Z);

Ey_xz  = E0*sin(pi*Xz/a).*phase;

Hx_xz  = -(E0/ZTE)*sin(pi*Xz/a).*phase;

Hz_xz  = (1j*pi*E0/(k0*eta0*a))*cos(pi*Xz/a).*phase;

figure('Name','TE10 mode','Position',[50 50 1100 700]);

subplot(2,2,1);

surf(X*1e3,Y*1e3,abs(Ey_xy)); shading interp; view(2); colorbar; axis tight;

title('TE_{10}: |E_y| over cross-section (one half-cycle across a)');

xlabel('x (mm)'); ylabel('y (mm)');

subplot(2,2,2);

quiver(X*1e3,Y*1e3,zeros(size(X)),real(Ey_xy),0.6,'r');

title('TE_{10}: E-field vectors (along y)'); xlabel('x (mm)'); ylabel('y (mm)'); axis tight;

subplot(2,2,3);

pcolor(Z*1e3,Xz*1e3,real(Ey_xz)); shading interp; colorbar;

title('TE_{10}: Re(E_y) in x-z plane (guide wavelength along z)');

xlabel('z (mm)'); ylabel('x (mm)');

subplot(2,2,4);

step = 6;

quiver(Z(1:step:end,1:step:end)*1e3, Xz(1:step:end,1:step:end)*1e3, ...

real(Hz_xz(1:step:end,1:step:end)*(-1j)), real(Hx_xz(1:step:end,1:step:end)), 1.5,'b');

title('TE_{10}: H-field loops in x-z plane (H_x, H_z)');

xlabel('z (mm)'); ylabel('x (mm)'); axis tight;

%% 6. TM11 FIELDS  (Ez, Ex, Ey, Hx, Hy)  - evanescent at 10 GHz

bt = beta_TM11_th;                    % = -j*alpha at 10 GHz

m = 1; n = 1;

Ez = E0*sin(m*pi*X/a).*sin(n*pi*Y/b);

Ex = -1j*bt/kc_TM11^2*(m*pi/a)*E0*cos(m*pi*X/a).*sin(n*pi*Y/b);

Ey = -1j*bt/kc_TM11^2*(n*pi/b)*E0*sin(m*pi*X/a).*cos(n*pi*Y/b);

Hx =  1j*w*eps0/kc_TM11^2*(n*pi/b)*E0*sin(m*pi*X/a).*cos(n*pi*Y/b);

Hy = -1j*w*eps0/kc_TM11^2*(m*pi/a)*E0*cos(m*pi*X/a).*sin(n*pi*Y/b);

% normalise phase so vectors can be drawn (common phase factor)

[~,iE] = max(abs(Ex(:)));  pE = exp(-1j*angle(Ex(iE)));

[~,iH] = max(abs(Hx(:)));  pH = exp(-1j*angle(Hx(iH)));

figure('Name','TM11 mode','Position',[80 80 1100 700]);

subplot(2,2,1);

surf(X*1e3,Y*1e3,abs(Ez)); shading interp; view(2); colorbar; axis tight;

title('TM_{11}: |E_z| (one variation along a and along b)');

xlabel('x (mm)'); ylabel('y (mm)');

subplot(2,2,2);

contour(X*1e3,Y*1e3,Ez,15); hold on;

quiver(X*1e3,Y*1e3,real(Ex*pE),real(Ey*pE),1.2,'r');

title('TM_{11}: E_z contours + transverse E vectors'); xlabel('x (mm)'); ylabel('y (mm)'); axis tight;

subplot(2,2,3);

quiver(X*1e3,Y*1e3,real(Hx*pH),real(Hy*pH),1.2,'b');

title('TM_{11}: transverse H vectors (circulate around E_z peak)');

xlabel('x (mm)'); ylabel('y (mm)'); axis tight;

subplot(2,2,4);

zz = linspace(0,L,200);

plot(zz*1e3, exp(imag(bt)*zz),'k','LineWidth',1.5); grid on;

title(sprintf('TM_{11} at %.0f GHz: field decay e^{-\\alpha z}, \\alpha = %.1f Np/m',f/1e9,abs(imag(bt))));

xlabel('z (mm)'); ylabel('Normalised amplitude');

%% 7. TEM FIELDS IN COAX (Er, Hphi)

rr = linspace(ri,ro,40);  ph = linspace(0,2*pi,73);

[R,PH] = meshgrid(rr,ph);

Xc = R.*cos(PH);  Yc = R.*sin(PH);

V0 = 1;

Er   = V0./(R*log(ro/ri));

Hphi = Er/eta0;

Ex_c = Er.*cos(PH);        Ey_c = Er.*sin(PH);

Hx_c = -Hphi.*sin(PH);     Hy_c = Hphi.*cos(PH);

figure('Name','TEM mode (coax)','Position',[110 110 1100 450]);

subplot(1,2,1);

pcolor(Xc*1e3,Yc*1e3,Er); shading interp; colorbar; hold on;

s = 3;

quiver(Xc(1:s:end,1:s:end)*1e3,Yc(1:s:end,1:s:end)*1e3,Ex_c(1:s:end,1:s:end),Ey_c(1:s:end,1:s:end),1.0,'r');

axis equal tight; title('TEM (coax): E-field radial'); xlabel('x (mm)'); ylabel('y (mm)');

subplot(1,2,2);

pcolor(Xc*1e3,Yc*1e3,Hphi); shading interp; colorbar; hold on;

quiver(Xc(1:s:end,1:s:end)*1e3,Yc(1:s:end,1:s:end)*1e3,Hx_c(1:s:end,1:s:end),Hy_c(1:s:end,1:s:end),1.0,'b');

axis equal tight; title('TEM (coax): H-field circumferential'); xlabel('x (mm)'); ylabel('y (mm)');

%% 8. PROPAGATION CONSTANT VS FREQUENCY (cutoff behaviour of all three modes)

fs = linspace(5e9,18e9,600);

bTE = zeros(size(fs)); bTM = bTE; aTE = bTE; aTM = bTE;

for i = 1:numel(fs)

k  = 2*pi*fs(i)/c;

t1 = getBeta(k,kc_TE10);  t2 = getBeta(k,kc_TM11);

bTE(i) = real(t1);  aTE(i) = abs(imag(t1));

bTM(i) = real(t2);  aTM(i) = abs(imag(t2));

end

bTEM = 2*pi*fs/c;

figure('Name','Dispersion','Position',[140 140 1000 420]);

subplot(1,2,1);

plot(fs/1e9,bTEM,'k--',fs/1e9,bTE,'b',fs/1e9,bTM,'r','LineWidth',1.5); grid on; hold on;

xline(f/1e9,'g--','10 GHz');

xline(fc_TE10_th/1e9,'b:'); xline(fc_TM11_th/1e9,'r:');

legend('TEM (\beta = k_0)','TE_{10}','TM_{11}','Location','northwest');

xlabel('Frequency (GHz)'); ylabel('\beta (rad/m)'); title('Phase constant (real part)');

subplot(1,2,2);

plot(fs/1e9,aTE,'b',fs/1e9,aTM,'r','LineWidth',1.5); grid on; hold on;

xline(f/1e9,'g--');

legend('TE_{10}','TM_{11}'); ylim([0 600]);

xlabel('Frequency (GHz)'); ylabel('\alpha (Np/m)'); title('Attenuation below cutoff (evanescent)');

%% ======================= LOCAL FUNCTIONS ================================

function bt = getBeta(k, kc)

% beta real above cutoff; beta = -j*alpha below cutoff (decaying e^{-j*beta*z})

if k > kc

bt = sqrt(k^2 - kc^2);

else

bt = -1j*sqrt(kc^2 - k^2);

end

end

function kc2 = fdEig(a,b,Nx,Ny,bc)

% Cell-centred finite-difference eigenvalues of -Laplacian on a x b rectangle

dx = a/Nx;  dy = b/Ny;

Dx = lap1d(Nx,bc)/dx^2;

Dy = lap1d(Ny,bc)/dy^2;

A  = -(kron(speye(Ny),Dx) + kron(Dy,speye(Nx)));

kc2 = sort(real(eig(full(A))));

end

function D = lap1d(N,bc)

e = ones(N,1);

D = spdiags([e -2*e e],-1:1,N,N);

if strcmp(bc,'dirichlet')

D(1,1) = -3;  D(N,N) = -3;

else

D(1,1) = -1;  D(N,N) = -1;

end

end

function s = ternary(cond,a,b)

if cond, s = a; else, s = b; end

end

OUTPUT:

=========== RESULTS AT f = 10.0 GHz ===========
Coax Z0 = 49.94 ohm, TEM: fc = 0, beta = k0 = 209.58 rad/m

Parameter                          Theory    Simulated     %% Dev
fc TE10 (GHz)                      6.5571       6.5565      0.010
fc TM11 (GHz)                     16.1451      16.1382      0.042
beta TE10 (rad/m)                 158.238      158.250      0.008
alpha TM11 (Np/m)                 265.655      265.472      0.069

TE10 : f > fc -> PROPAGATES (beta real)
TM11 : f < fc -> EVANESCENT (beta imaginary)
Deviation limit <= 5% : MET