clc;

clear;

close all;

% Electromagnetic Simulation and S-Parameter Characterization

% Microwave Directional Coupler and Two-Way Power Divider

%% Simulation Parameters

f0 = 10e9;                         % Center frequency (Hz)

BW = 2e9;                          % Bandwidth (Hz)

f = linspace(9e9, 11e9, 201);      % Frequency range (Hz)

x = (f - f0) / (BW/2);

Z0 = 50;                           % Characteristic impedance (Ohm)

c = 3e8;                           % Speed of light (m/s)

lambda0 = c / f0;                  % Wavelength at center frequency

L = lambda0 / 4;                   % Quarter-wave length

%% 1. Directional Coupler

% Coupling level in dB

coupling_dB = -10 + cos(pi*x);

% Convert coupling from dB to linear magnitude

k = 10.^(coupling_dB / 20);

% Ensure k does not exceed 1

k = min(k, 0.999);

% Through coefficient

through = sqrt(1 - k.^2);

% Phase constant

beta = 2*pi*f/c;

% S-parameters

S11_c = 0.02 * ones(size(f));

S21_c = through .* exp(-1j * beta * L);

S31_c = k .* exp(-1j * (beta*L + pi/2));

S41_c = 10^(-35/20) * ones(size(f));

% Convert S-parameters to dB

S11c_dB = 20*log10(abs(S11_c));

S21c_dB = 20*log10(abs(S21_c));

S31c_dB = 20*log10(abs(S31_c));

S41c_dB = 20*log10(abs(S41_c));

%% 2. Two-Way Power Divider

S11_d = 0.02 + 0.16*abs(x);

S21_d = (1/sqrt(2)) .* (1 - 0.04*x.^2);

S31_d = (1/sqrt(2)) .* (1 - 0.04*x.^2);

S23_d = 0.01 + 0.14*abs(x);

% Convert S-parameters to dB

S11d_dB = 20*log10(abs(S11_d));

S21d_dB = 20*log10(abs(S21_d));

S31d_dB = 20*log10(abs(S31_d));

S23d_dB = 20*log10(abs(S23_d));

%% 3. Numerical Results

[~, idx] = min(abs(f - f0));

fprintf('\n');

fprintf('MICROWAVE DEVICE SIMULATION RESULTS\n');

fprintf('--------------------------------------\n');

fprintf('Characteristic Impedance = %.1f Ohms\n', Z0);

fprintf('Center Frequency         = %.2f GHz\n', f(idx)/1e9);

fprintf('Wavelength               = %.2f mm\n', lambda0*1000);

fprintf('Quarter-Wave Length      = %.2f mm\n', L*1000);

fprintf('\nDIRECTIONAL COUPLER\n');

fprintf('-------------------\n');

fprintf('S11 = %.2f dB\n', S11c_dB(idx));

fprintf('S21 = %.2f dB\n', S21c_dB(idx));

fprintf('S31 = %.2f dB\n', S31c_dB(idx));

fprintf('S41 = %.2f dB\n', S41c_dB(idx));

fprintf('\nTWO-WAY POWER DIVIDER\n');

fprintf('---------------------\n');

fprintf('S11 = %.2f dB\n', S11d_dB(idx));

fprintf('S21 = %.2f dB\n', S21d_dB(idx));

fprintf('S31 = %.2f dB\n', S31d_dB(idx));

fprintf('S23 = %.2f dB\n', S23d_dB(idx));

%% 4. Combined Subplot Figure

figure('Name', 'Microwave S-Parameter Characterization', ...

'NumberTitle', 'off');

%% Subplot 1: Directional Coupler

subplot(3,1,1);

plot(f/1e9, S11c_dB, 'LineWidth', 1.5);

hold on;

plot(f/1e9, S21c_dB, 'LineWidth', 1.5);

plot(f/1e9, S31c_dB, 'LineWidth', 1.5);

plot(f/1e9, S41c_dB, 'LineWidth', 1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('Directional Coupler S-Parameters');

legend('S11', 'S21', 'S31', 'S41', ...

'Location', 'best');

ylim([-45 2]);

%% Subplot 2: Two-Way Power Divider

subplot(3,1,2);

plot(f/1e9, S11d_dB, 'LineWidth', 1.5);

hold on;

plot(f/1e9, S21d_dB, 'LineWidth', 1.5);

plot(f/1e9, S31d_dB, 'LineWidth', 1.5);

plot(f/1e9, S23d_dB, 'LineWidth', 1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('Two-Way Power Divider S-Parameters');

legend('S11', 'S21', 'S31', 'S23', ...

'Location', 'best');

ylim([-45 2]);

%% Subplot 3: Power Distribution

subplot(3,1,3);

plot(f/1e9, abs(S21_c).^2 * 100, ...

'LineWidth', 1.5);

hold on;

plot(f/1e9, abs(S31_c).^2 * 100, ...

'LineWidth', 1.5);

plot(f/1e9, abs(S21_d).^2 * 100, '--', ...

'LineWidth', 1.5);

plot(f/1e9, abs(S31_d).^2 * 100, '--', ...

'LineWidth', 1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Power (%)');

title('Microwave Power Distribution');

legend('Coupler Through', ...

'Coupler Coupled', ...

'Divider Output 1', ...

'Divider Output 2', ...

'Location', 'best');

%% Overall Title

sgtitle('Microwave Directional Coupler and Two-Way Power Divider');